Capstone Report

Manav Patel 0301184p

AllerScan

Supervisor: Dr. Lydia Bouzar-Benlabiod

- Abstract

This report presents the development of AllerScan, an AI-driven mobile application designed to enhance allergen detection in food products. The project aims to address the challenges faced by individuals with allergies by improving the accuracy of allergen detection, with support for both English and French ingredient labels. By leveraging advanced Optical Character Recognition (OCR) techniques and personalized allergen filtering, AllerScan provides users with reliable and tailored allergen information.

The application employs a multi-strategy OCR approach, utilizing various Page Segmentation Modes (PSM) and Optical Engine Modes (OEM) to optimize text recognition. Character whitelisting and language-aware text normalisation reduce OCR noise, while curated bilingual allergen dictionaries keep detection precise. Additionally, the system incorporates a comprehensive pattern detection mechanism to identify potential allergens listed under "Your Allergens" sections.

The results demonstrate a significant improvement in allergen detection accuracy, offering a valuable tool for users to make informed food choices. AllerScan’s provider-driven localisation, emoji-only onboarding, and purely on-device workflow ensure a seamless experience that increases user confidence and safety.

- Introduction

  - Background

Food allergies are a significant health concern, affecting millions of individuals worldwide. Accurate labeling and detection of allergens in food products are crucial for consumer safety, as even trace amounts of allergens can trigger severe reactions. Traditional label review breaks down when layouts shift across brands or languages, leaving critical warnings easy to miss.

  - Objective

The primary objective is to deliver AllerScan, an offline Flutter application that scans ingredient labels, normalizes English/French text, and highlights profile-specific allergens so users can decide quickly.

  - Importance

AllerScan addresses a critical need for enhanced allergen detection by leveraging advanced AI technologies. By combining on-device OCR, curated bilingual dictionaries, and instant localisation, the app boosts user confidence while keeping sensitive data private.

- Literature Review

  - Existing Solutions

The detection and labeling of allergens in food products have traditionally relied on manual processes and standard labeling practices. These methods often lack the precision needed to identify allergens listed under ambiguous "may contain" sections. Current tools rarely reconcile cross-contamination warnings or bilingual labelling, so users still rely on manual review.

  - AI in Allergen Detection

Recent advancements in Artificial Intelligence (AI) have opened new avenues for improving allergen detection accuracy. AI techniques, such as Optical Character Recognition (OCR), have been employed to automate the extraction of textual information from food labels. OCR pipelines help automate extraction, yet their accuracy drops when fonts, languages, or layouts shift between brands.

  - Challenges and Opportunities

The key challenge is pulling reliable signals from noisy, mixed-language ingredient lists. AllerScan addresses this by combining multi-pass OCR, language-aware text normalisation, and curated English/French allergen dictionaries so user-specific allergens are highlighted consistently.

- Methodology

  - Approach

The AllerScan project employs a multi-strategy Optical Character Recognition (OCR) approach to enhance the detection of allergens in food labels. The pipeline captures a label image, runs multiple on-device OCR passes tuned for English and French packaging, normalises the text, and matches the results against the user’s saved allergen list.

  - Tools and Technologies

- Framework: Flutter (Android build) with Dart 3.

- OCR Engine: ‘flutter_tesseract_ocr’ (bundled tessdata configs per language).”

- Image handling: ‘image_picker’ plus ‘crop_your_image’ for capture and preprocessing.”

- State and Localisation: Provider-driven, ‘LanguageProvider’ with ‘shared_preferences’ persistence.

- Text normalisation: ‘characters’ package and custom helpers for bilingual cleanup.

  - Techniques

- Multi-pass OCR (PSM/OEM combos per language) to improve recognition on varied layouts.

- Character whitelisting and language-aware normalisation to strip OCR noise.”

- Dictionary matching against curated English/French JSON allergen lists, filtered by profile.

- Confidence heuristics to prioritise high-certainty matches and drop duplicates.

  - Implementation Steps

- Image Acquisition: Users capture or select images of food labels for analysis.

- Image Preprocessing: Images are cropped and prepared for OCR processing.

- Text Recognition: OCR engine extracts text using multiple configurations to ensure accuracy.

- Allergen Detection: Extracted text is normalised then with the appropriate language dictionary and surface confirmed matches from the user’s profile.

- User-Specific Filtering: Results are filtered based on user profiles to provide personalized allergen information.

- Implementation

  - System Architecture

The AllerScan application is structured to provide a seamless user experience, integrating advanced OCR capabilities with a user-friendly interface. The architecture consists of the following components:

- Frontend: Flutter screens (Onboarding, Home, Upload, Results, Profile) driven by a ‘LanguageProvider’ for instant EN/FR updates.

- On-device services: ‘flutter_tesseract_ocr’ handles text extraction; Dart utilities (text normalisation, allergen detector, ‘ProfileService’) process results and persist user data via ‘SharedPreferences’.

  - Key Features

- Multi-Strategy OCR: Multiple PSM/OEM combinations per language ensure reliable recognition even when label layouts differ.

- Noise reduction & normalisation: Character whitelisting plus bilingual typo mapping clean OCR output before matching.

- Curated allergen dictionaries: English/French JSON sources are filtered through the user’s saved allergen list so only relevant hits appear.

- Localised presentation: Provider-driven strings and allergen labels keep the UI consistent when users switch languages or reset their profile.

  - Screenshots

  - Integration with User Profiles

AllerScan stores each user’s allergens, custom entries, language choice, and emoji avatar in SharedPreferences via ProfileService. Onboarding saves the profile; the profile screen updates it and deleting it clears the data and resets the language to English.

- Results

  - Findings

Manual testing across English and French labels showed that the multi-pass OCR pipeline consistently surfaced confirmed allergens from the user’s profile while filtering out text that wasn’t relevant, cutting down false positives.

  - Performance Metrics

- Detection consistency: The combined OCR + normalization pipeline reliably extracted ingredients across differently formatted labels.

- Response time: A typical scan (capture → OCR → match) finished within a few seconds on mid-range Android hardware.

- User feedback: Early testers highlighted the localized UI and straightforward allergen alerts as the main usability wins.

  - Comparison with Initial Objectives

AllerScan met its objectives: it delivers accurate, profile-specific allergen flags, keeps everything on-device, and localizes the entire experience through the provider-based language system.

- Discussion

  - Interpretation of Results

AllerScan’s multi-pass OCR, language-aware normalisation, and curated allergen dictionaries consistently highlighted the profile-specific allergens we set out to detect, validating the offline-first approach and the localisation work.

  - Challenges Faced

Several challenges were encountered during the development of AllerScan. The main hurdles were tuning OCR configs for mixed-language packaging, keeping dictionaries in sync with the current locale, and hardening the provider so language switches persisted reliably. Integrating multiple OCR configurations required careful calibration to balance accuracy and processing speed. Additionally, ensuring compatibility with various label designs and text presentations posed technical challenges that were addressed through iterative testing and refinement.

  - Areas of Improvement

While the project has met its primary goals, there are opportunities for further enhancement. Future work could explore the integration of additional languages to broaden the application's accessibility. Next iterations should add more locales, richer profile metadata (e.g., severity notes), and automated widget/unit tests so regressions are caught early.

  - Implications of Future Research

The success of AllerScan highlights the potential for AI-driven solutions to address complex challenges in food safety. Future research could explore the application of similar techniques to other areas of consumer safety, such as detecting contaminants in food or identifying allergens in non-food products.

- Conclusion

AllerScan demonstrates that a lightweight, offline Flutter application can reliably scan packaged-food labels, normalize English and French text, and surface profile-specific allergens with minimal latency. By combining multi-pass Tesseract OCR configurations, language-aware text normalisation, and curated allergen dictionaries, the project met its core technical goal: delivering trustworthy allergen detection on-device without exposing sensitive data to external services.

The user experience evolved alongside the technical pipeline. Emoji-only avatars simplified onboarding, the provider-driven localisation framework kept the interface responsive to language changes, and the removal of the confusing “May Contain” section aligned the results screen with stakeholder feedback. Together, these refinements ensured the app stayed approachable while still offering nuanced allergen insights.

Future work will focus on widening the feature set: expanding the dictionaries beyond English/French, supporting richer profile metadata (e.g., severity levels or household sharing), adding automated tests, and exploring secure sync so families can keep their allergen preferences aligned.

Overall, AllerScan delivers a dependable, bilingual allergen scanner for Android users and lays a solid foundation for a broader suite of consumer food-safety tools.